# SIMATS ENGINEERING

# Department of Computer Science & Engineering

ASSESSMENT TOOL 1- EXPERIMENTS

| Course Code: | CSA1205 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO3 | Assessment: | ASSESSMENT TOOL 1- EXPERIMENTS |
| Weightage: | 40% | Total Marks: | 25 |
| CO3 Statement: | Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT). (BL6) |  |  |
| Student Name: | Kavinadhithya R S | Reg.No.: | 192511365 |
| Department: | B.E. CSE | Date: |  |

# ANNEXURE A

- Design and simulate a system that performs register transfer operations along with basic arithmetic and logic operations such as addition, subtraction, AND, and OR. Use multiple registers to demonstrate how data is transferred between them during execution, and analyze the sequence of operations and the role of the ALU in processing data.

- Create a model of a computer system using single-bus and multi-bus organization, and perform data transfer operations between registers, memory, and I/O. Compare the time taken and efficiency of both approaches, and analyze how multiple bus structures improve parallel data transfer and reduce bottlenecks.

- Simulate a 5-stage instruction pipeline (Fetch, Decode, Execute, Memory, Write-back) and execute a set of instructions. Measure the performance in terms of speedup and throughput, and compare it with a non-pipelined system to evaluate the effectiveness of pipelining.

- Develop a pipeline execution scenario where data hazards, control hazards, and structural hazards occur. Observe their impact on instruction execution, and implement techniques such as stalling, data forwarding, and branch prediction to resolve these hazards and improve performance.

- Analyze a processor supporting superscalar execution, including out-of-order and speculative execution, and extend the study to include hyper-threading and simultaneous multithreading (SMT). Evaluate how multiple instructions are executed in parallel and compare the performance improvements in terms of instruction throughput and resource utilization.

Code of Conduct:

I 	(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

# MARKS ALLOCATION

|  | Understanding of Concepts (20%) | Experimental Design (20%) | Use of Tools (25%) | Analysis of Results (20%) | Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

# RUBRICS FOR CALCULATION:

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |

### Register Transfer and ALU Operations Aim

To design and analyze a system that performs register transfer operations along with basic arithmetic and logic operations such as addition, subtraction, AND, and OR using multiple registers and an ALU.

### Introduction

A register transfer operation is the movement of data from one register to another. The Arithmetic Logic Unit (ALU) performs arithmetic and logical operations on the data stored in registers. The Control Unit generates signals to select the required operation.

### Block Diagram

### Register Organization

Assume four 8-bit registers:

| Register | Function |
| --- | --- |
| R0 | First operand |
| R1 | Second operand |
| R2 | ALU result |
| R3 | Temporary register |

Initial values:

R0 = 00001111 = 15

R1 = 00000011 = 3

### Register Transfer

A register transfer is represented as:

R2 ← R0

This means the contents of R0 are transferred to R2. Example: R3 ← R0

Therefore:

R3 = 00001111

### ALU Operations

- Addition R2 ← R0 + R1

00001111

+ 00000011

00010010

R2 = 18

- Subtraction R2 ← R0 − R1

00001111

- 00000011

00001100

R2 = 12

## AND

R2 ← R0 AND R1

00001111

00000011

00000011

R2 = 3

## OR

R2 ← R0 OR R1

00001111

00000011

00001111

R2 = 15

### Operation Sequence

| Step | Operation | Result |
| --- | --- | --- |
| 1 | R0 ← 15 | R0 = 15 |
| 2 | R1 ← 3 | R1 = 3 |
| 3 | R3 ← R0 | R3 = 15 |
| 4 | R2 ← R0 + R1 | R2 = 18 |
| 5 | R2 ← R0 − R1 | R2 = 12 |
| 6 | R2 ← R0 AND R1 | R2 = 3 |
| 7 | R2 ← R0 OR R1 | R2 = 15 |

### Role of ALU

The ALU performs the required arithmetic and logical operations. The Control Unit selects the operation and the registers provide the operands.

Data flow:

Source Registers → ALU → Destination Register

The system successfully performs register transfers and addition, subtraction, AND, and OR operations. The role of the ALU and the sequence of data transfers between registers were analyzed successfully.

### Single-Bus and Multi-Bus Organization Aim

To model and compare single-bus and multi-bus computer organizations for data transfer between registers, memory, and I/O, and analyze their performance.

### Introduction

A bus is a communication path used to transfer data between different components of a computer. In a single-bus system, all components share one bus, whereas a multi-bus system uses multiple buses to allow parallel transfers.

### Single-Bus Organization

All registers, ALU, memory, and I/O devices share one common bus. R0 ──┐

R1 ──┤

R2 ──┼──► SINGLE BUS ──► ALU Memory┤ │

I/O ─┘	▼

Destination Example:

R1 ← R2

R2 → Bus → R1 For:

R3 ← R1 + R2

the operands must be transferred sequentially through the common bus.

### Limitation

Since only one major transfer can use the bus at a time, the bus becomes a bottleneck.

### Multi-Bus Organization

A multi-bus system uses multiple buses, allowing different data transfers to occur simultaneously.

R1 ──► Bus A ──┐

▼

ALU ──► Bus C ──► R3

▲

R2 ──► Bus B ──┘ For:

R3 ← R1 + R2

R1 and R2 can be transferred to the ALU simultaneously through Bus A and Bus B.

### Register, Memory and I/O Transfers Register Transfer

R1 ← R2

R2 → Bus → R1 Memory Transfer R1 ← Memory

Memory → Bus → R1

I/O Transfer R1 ← I/O

I/O → Bus → R1

In a multi-bus system, independent transfers can use different buses at the same time.

### Comparison

| Feature | Single Bus | Multi Bus |
| --- | --- | --- |
| Number of buses | One | Multiple |
| Data transfer | Sequential | Parallel |
| Speed | Lower | Higher |
| Hardware complexity | Low | High |
| Cost | Low | High |
| Bus bottleneck | High | Reduced |
| Parallelism | Limited | High |
| ALU utilization | Lower | Higher |

### Performance Example

Assume a particular sequence requires:

- Single-bus system = 7 clock cycles

- Multi-bus system = 4 clock cycles

Speedup:

Therefore, the multi-bus system is 1.75 times faster for this example.

### Advantages of Multi-Bus Organization

- Allows parallel data transfer.

- Reduces bus contention.

- Reduces execution time.

- Improves ALU utilization.

- Increases overall throughput.

- Reduces communication bottlenecks.

### Result

The single-bus and multi-bus organizations were successfully analyzed. The multi-bus structure provides better performance because multiple data transfers can occur in parallel.

### Conclusion

A single-bus organization is simple and economical but suffers from a shared-bus bottleneck. A multi-bus organization is more complex and costly, but it improves parallelism, transfer speed, ALU utilization, and overall processor efficiency.

### Five-Stage Instruction Pipeline Aim

To simulate a 5-stage instruction pipeline consisting of Fetch, Decode, Execute, Memory, and Write-back stages, and compare its performance with a non-pipelined processor.

### Introduction

Pipelining is a technique in computer architecture where the execution of multiple instructions is overlapped. Instead of completing one instruction before starting the next, different instructions are processed in different stages simultaneously.

The five stages are:

  - IF – Instruction Fetch

  - ID – Instruction Decode

  - EX – Execute

  - MEM – Memory Access

  - WB – Write Back

### Pipeline Stages

| Stage | Function |
| --- | --- |
| IF | Fetches instruction from memory |
| ID | Decodes instruction and reads registers |
| EX | Performs ALU operation |
| MEM | Accesses data memory |
| WB | Writes result back to register |

### Example Instructions

Consider four instructions:

    - I1: R1 ← R2 + R3

    - I2: R4 ← R5 − R6

    - I3: R7 ← R1 AND R8

    - I4: R9 ← R10 OR R11

### Pipeline Execution

| Clock Cycle | I1 | I2 | I3 | I4 |
| --- | --- | --- | --- | --- |
| 1 | IF |  |  |  |
| 2 | ID | IF |  |  |
| 3 | EX | ID | IF |  |
| 4 | MEM | EX | ID | IF |
| 5 | WB | MEM | EX | ID |
| 6 |  | WB | MEM | EX |
| 7 |  |  | WB | MEM |
| 8 |  |  |  | WB |

Thus, four instructions are completed in 8 clock cycles. Non-Pipelined Execution

In a non-pipelined processor, each instruction completes all five stages before the next instruction starts.

For 4 instructions:

Therefore:

    - Non-pipelined = 20 cycles

    - Pipelined = 8 cycles Speedup

Therefore, the pipelined processor provides a 2.5× speedup for this example.

### Throughput

Throughput refers to the number of instructions completed per unit time.

After the pipeline is filled, approximately one instruction can be completed every clock cycle. Thus, pipelining significantly improves instruction throughput.

### Advantages

    - Increases instruction throughput.

    - Improves processor resource utilization.

    - Allows multiple instructions to execute simultaneously at different stages.

    - Reduces the average time per instruction.

### Result

The 5-stage instruction pipeline was analyzed successfully. Compared with a non-pipelined processor, pipelining reduces execution time and increases instruction throughput.

### Conclusion

Instruction pipelining improves processor performance by overlapping the execution of multiple instructions. Although the first instruction still requires five stages, subsequent instructions can progress simultaneously through different stages, resulting in higher throughput.

### Pipeline Hazards and Their Resolution Aim

To analyze data, control, and structural hazards in a pipelined processor and study techniques such as stalling, data forwarding, and branch prediction for resolving these hazards.

### Introduction

A pipeline hazard is a condition that prevents the next instruction from executing in its normal clock cycle.

The three major types are:

- Data Hazard

- Control Hazard

- Structural Hazard

- Data Hazard

A data hazard occurs when an instruction depends on the result of a previous instruction. Example:

I1: R1 ← R2 + R3 I2: R4 ← R1 + R5

Here, I2 needs the value of R1 produced by I1.

### Problem

I2 may try to read R1 before I1 has written the result.

### Solution: Data Forwarding

The result from the EX stage of I1 is directly forwarded to the EX stage of I2. This reduces unnecessary waiting.

### Control Hazard

A control hazard occurs due to a branch or jump instruction. Example:

I1: BEQ R1, R2, LABEL I2: ADD R3, R4, R5

The processor does not immediately know whether I2 should be executed because the branch decision is not yet available.

### Solution: Branch Prediction

The processor predicts whether the branch will be:

  - Taken, or

### Not taken

If the prediction is correct, execution continues without major delay.

If the prediction is incorrect, the incorrectly fetched instructions are removed from the pipeline.

### Structural Hazard

A structural hazard occurs when two pipeline stages require the same hardware resource at the same time.

Example:

Instruction 1 → Memory access Instruction 2 → Instruction fetch

If the processor has only one memory unit, both operations cannot occur simultaneously.

### Solution: Stalling

The processor temporarily pauses one instruction until the required resource becomes available. Cycle: 1 2 3 4 5

I1:	IF ID EX MEM WB I2:		IF ID ST EX

ST = Stall

### Hazard Summary

| Hazard | Cause | Example | Solution |
| --- | --- | --- | --- |
| Data | Data dependency | I2 uses I1 result | Forwarding / Stalling |
| Control | Branch instruction | Conditional branch | Branch prediction |
| Structural | Resource conflict | Same memory used twice | Stalling / Extra hardware |

### Effect of Hazards

Without hazard handling, the pipeline may produce incorrect results or require additional waiting cycles.

Hazards can:

- Increase execution time.

- Reduce throughput.

- Create pipeline stalls.

- Reduce processor efficiency.

### Hazard Resolution Techniques

- Stalling

The processor inserts a delay or bubble into the pipeline.

Advantage: Simple and reliable.

Disadvantage: Increases execution time.

### Data Forwarding

The required result is directly passed from one pipeline stage to another without waiting for write-back.

Advantage: Reduces data-related stalls.

### Branch Prediction

The processor predicts the outcome of a branch before the actual result is known.

Advantage: Reduces control-hazard delays.

### Overall Pipeline Flow

Branch prediction helps with control hazards, while stalling and forwarding help handle data and structural hazards.

### Result

Data, control, and structural hazards were analyzed successfully. Stalling, data forwarding, and branch prediction were identified as techniques to reduce the performance loss caused by pipeline hazards.

### Conclusion

Pipeline hazards can reduce the performance of a pipelined processor. Data forwarding reduces data dependency delays, branch prediction reduces control-hazard penalties, and stalling resolves situations where an operation cannot safely proceed. These techniques improve pipeline efficiency and maintain higher instruction throughput.

### 5. Superscalar, Out-of-Order, Speculative Execution and SMT Aim

To analyze a processor supporting superscalar execution, out-of-order execution, speculative execution, hyper-threading, and simultaneous multithreading (SMT) and evaluate their effect on instruction throughput and resource utilization.

### Introduction

A superscalar processor can execute more than one instruction in a single clock cycle by using multiple execution units.

Out-of-order execution allows instructions to execute when their required operands and resources are available, rather than strictly following program order.

Speculative execution allows the processor to execute instructions before it is certain that they are required, usually based on branch prediction.

Hyper-threading/SMT allows multiple instruction streams or threads to share the resources of a single physical processor core.

### Superscalar Execution

A superscalar processor contains multiple execution units.

For example, a 2-way superscalar processor may execute two independent instructions per clock cycle.

### Out-of-Order Execution

In normal in-order execution:

I1 → I2 → I3 → I4

In out-of-order execution, if I2 is waiting for data but I3 is ready: I1 → I2 (waiting)

↓

I3 → I4

Thus, ready instructions can execute first.

### Advantages

- Reduces processor idle time.

- Improves execution-unit utilization.

- Increases instruction-level parallelism.

- Improves overall throughput.

### Speculative Execution

Speculative execution occurs when the processor predicts the result of a branch and executes instructions along the predicted path before the branch is confirmed.

If the prediction is correct, useful work has already been completed.

If the prediction is incorrect, the speculative results are discarded and the correct instructions are executed.

### Hyper-Threading / SMT

Simultaneous Multithreading (SMT) allows multiple threads to share the resources of a single processor core.

When one thread is waiting for memory or another resource, another thread can use the available execution resources.

### Example of Parallel Execution

Consider four independent instructions:

### Instruction Operation

| I1 | R1 ← R2 + R3 |
| --- | --- |
| I2 | R4 ← R5 − R6 |
| I3 | R7 ← R8 AND R9 |
| I4 | R10 ← R11 OR R12 |

A single-issue processor may execute approximately one instruction at a time. A 2-way superscalar processor can execute:

### Cycle Execution

1	I1, I2

2	I3, I4

Thus, four instructions can be completed in approximately 2 execution cycles, assuming no dependencies or resource conflicts.

### Performance Comparison

Assume 8 instructions need to be executed.

| Processor Type | Instructions/Cycle | Approx. Cycles |
| --- | --- | --- |
| Single-issue | 1 | 8 |
| 2-way superscalar | 2 | 4 |
| 4-way superscalar | 4 | 2 |

For a 2-way superscalar processor:

Therefore, the ideal speedup is 2×.

In practice, the speedup is lower because of:

  - Data dependencies

  - Branches

  - Cache misses

  - Limited execution units

  - Resource conflicts

### Effect on Resource Utilization

| Technique | Main Benefit |
| --- | --- |
| Superscalar | Executes multiple instructions simultaneously |
| Out-of-order | Uses idle execution units efficiently |
| Speculative execution | Reduces branch waiting time |
| SMT/Hyper-threading | Uses unused resources between threads |

These techniques work together to increase processor utilization and instruction throughput.

### Overall Comparison

| Feature | Basic Processor | Advanced Processor |
| --- | --- | --- |
| Instruction issue | One at a time | Multiple |

| Feature | Basic Processor | Advanced Processor |
| --- | --- | --- |
| Execution order | Mostly in order | Out of order |
| Branch handling | Normal | Speculative |
| Threads/core | Usually one | Multiple |
| Parallelism | Low | High |
| Throughput | Lower | Higher |
| Resource utilization | Lower | Higher |

### Result

The analysis shows that superscalar execution, out-of-order execution, speculative execution, and SMT significantly improve instruction-level and thread-level parallelism. Multiple instructions can be processed simultaneously, increasing instruction throughput and making better use of processor resources.

### Conclusion

A modern processor improves performance by combining multiple techniques. Superscalar execution increases the number of instructions executed per cycle, out-of-order execution keeps execution units busy, speculative execution reduces branch delays, and SMT/Hyper-threading allows multiple threads to share processor resources. Together, these techniques improve instruction throughput, parallelism, and resource utilization.