SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2 – COMPARATIVE ANALYSIS

ANSWER SHEET – CO4

NAME : Kavinadhithya R S

REG.NO : 192511365

Course Code: CSA12    Course Title: Computer Architecture

CO4: Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics.

# Question 1 – L1, L2, and L3 Cache Comparison

Modern processors use a multi-level cache hierarchy to bridge the speed gap between the CPU and main memory. Each level trades off speed, size, and cost differently, as compared below.

| Parameter | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Location | Built into each CPU core | Per core or shared by a core cluster | Shared across all cores on the die |
| Typical Size | 16 KB – 64 KB (per core) | 256 KB – 1 MB (per core) | 4 MB – 64 MB (shared) |
| Latency | ~1–4 CPU cycles (fastest) | ~10–20 CPU cycles | ~30–70 CPU cycles |
| Bandwidth | Highest (very wide, close to core) | High, but lower than L1 | Moderate; shared among cores reduces effective bandwidth per core |
| Throughput | Highest — serves nearly all load/store instructions directly | High, handles L1 misses efficiently | Lower than L1/L2, but far higher than main memory |
| Cost per bit | Highest (SRAM, most transistors per cell) | High (SRAM) | High but slightly denser SRAM design |

Impact on processor performance

- L1 cache, being closest to the execution units, minimizes stalls for the most frequently accessed data/instructions and is critical for single-cycle-level performance.

- L2 cache acts as a buffer for L1 misses, reducing the penalty of going all the way to main memory, and its larger size improves the hit rate for medium working sets.

- L3 cache, being shared, improves performance for multi-core workloads by allowing data sharing between cores without accessing slower DRAM, and it reduces overall memory traffic.

- Overall, as we move from L1 to L3, latency increases and per-access bandwidth decreases, but capacity increases — this graded hierarchy exploits the principle of locality of reference to keep the effective average memory access time low despite slow main memory.

# Question 2 – SRAM vs. DRAM

SRAM and DRAM are the two primary memory technologies used in cache and main memory design respectively, differing fundamentally in circuit design and performance.

| Parameter | SRAM (Static RAM) | DRAM (Dynamic RAM) |
| --- | --- | --- |
| Cell structure | 6 transistors per bit (flip-flop based); no capacitor | 1 transistor + 1 capacitor per bit |
| Latency | Very low (~1–2 ns) — no refresh needed | Higher (~50–100 ns) — refresh and precharge add delay |
| Bandwidth | High, due to simpler, faster access circuitry | Lower per-cell bandwidth, though achieved in bulk via wide buses/burst modes |
| Throughput | High — well suited to rapid, repeated cache accesses | Moderate — good for bulk sequential access, but refresh cycles reduce sustained throughput |
| Density / Cost | Low density, expensive per bit | High density, cheap per bit |
| Power | Higher static power (leakage), but no refresh power | Lower per-cell power but consumes refresh power continuously |
| Typical use | Cache memory (L1/L2/L3) | Main memory (system RAM) |

Evaluation

SRAM's simpler, refresh-free design gives it far lower latency and higher throughput, making it ideal for cache levels where speed is paramount despite its high cost and low density. DRAM's capacitor-based cell allows much higher density at lower cost, making it economical for large main memory capacities, but its need for periodic refresh and longer access path results in higher latency, which is why it sits below the cache hierarchy rather than replacing it.

# Question 3 – Virtual Memory (Paging) vs. Cache Memory

Both virtual memory paging and cache memory implement a form of hierarchical, locality-based memory management, but they operate at very different levels of the memory hierarchy.

| Parameter | Cache Memory | Virtual Memory (Paging) |
| --- | --- | --- |
| Purpose | Bridges speed gap between CPU and main memory | Bridges capacity gap between main memory (RAM) and secondary storage (disk/SSD) |
| Unit of transfer | Cache line/block (e.g., 32–128 bytes) | Page (typically 4 KB or larger) |
| Access latency on hit | Very low (nanoseconds — 1–70 cycles) | Low (RAM access speed, tens of ns) when page is resident |
| Access latency on miss | Goes to next cache level or main memory (tens to hundreds of ns) | Page fault — requires disk/SSD I/O, taking milliseconds (orders of magnitude slower) |
| Management | Managed by hardware (cache controller) | Managed by OS software with hardware (MMU/TLB) support |
| Effect on throughput | High — misses are relatively cheap, so throughput stays high | Excessive page faults ('thrashing') can drastically reduce system throughput |

Comparative analysis

Cache memory operates at nanosecond timescales and is entirely hardware-managed, making its miss penalty relatively small in comparison to a page fault. Virtual memory paging operates at a coarser granularity and its miss penalty (a page fault requiring disk access) is orders of magnitude larger, since it involves mechanical or flash storage latency and OS intervention. Consequently, cache design focuses on minimizing miss rate and access latency in hardware, while virtual memory design focuses on minimizing page faults through intelligent page replacement algorithms (e.g., LRU) to preserve system throughput.

# Question 4 – NAND Flash vs. DRAM

NAND Flash and DRAM serve different roles in the memory/storage hierarchy — one as persistent storage, the other as volatile working memory.

| Parameter | DRAM | NAND Flash |
| --- | --- | --- |
| Volatility | Volatile — loses data without power | Non-volatile — retains data without power |
| Latency | Very low (tens of nanoseconds) | Much higher (microseconds for read; even higher for write/erase) |
| Bandwidth | High, byte-addressable random access | Lower; optimized for block/page-level sequential access |
| Write endurance | Effectively unlimited write cycles | Limited write/erase cycles (wear leveling required) |
| Cost per bit | Higher than NAND Flash | Lower than DRAM, enabling large storage capacities cheaply |
| Role in hierarchy | Main memory — fast working storage for active processes | Secondary/persistent storage (SSD) — bulk, non-volatile storage |

Suitability for hierarchical memory systems

DRAM's low latency and unlimited endurance make it suitable for the main memory tier, where the CPU needs fast, repeated read/write access to active data. NAND Flash, despite its higher latency and limited write endurance, offers non-volatility and much lower cost per bit at high density, making it ideal for the storage tier (SSDs) that holds data persistently between power cycles. In a hierarchical system, DRAM sits above NAND Flash, with the OS/hardware moving data between them (e.g., paging, caching of disk blocks) to balance speed and capacity.

# Question 5 – MRAM vs. RRAM

MRAM (Magnetoresistive RAM) and RRAM (Resistive RAM) are emerging non-volatile memory technologies being explored as alternatives/supplements within the memory hierarchy.

| Parameter | MRAM | RRAM (ReRAM) |
| --- | --- | --- |
| Storage mechanism | Stores data via magnetic orientation of a magnetic tunnel junction (MTJ) | Stores data via resistance change in a metal-oxide dielectric layer (formation/rupture of conductive filaments) |
| Access time | Very fast (comparable to SRAM/DRAM, ~ns range) | Fast, but generally slightly higher latency than MRAM |
| Throughput | High — supports high-speed read/write suitable for cache-like use | Good throughput, though somewhat lower than MRAM in most implementations |
| Endurance | Very high write endurance (near DRAM-like) | Moderate-to-high endurance, generally lower than MRAM |
| Energy efficiency | Low read/write energy; efficient but MTJ switching needs moderate write current | Very low energy per switching event; simpler cell structure aids efficiency |
| Scalability/Density | Somewhat limited by MTJ fabrication complexity | Highly scalable, simple structure supports very high density |
| Non-volatility | Yes | Yes |

Performance evaluation

MRAM offers a superior combination of speed and endurance that approaches DRAM/SRAM performance, making it attractive for replacing volatile memory in latency-critical roles while adding non-volatility. RRAM, while slightly behind MRAM in raw speed and endurance, offers a simpler, more scalable cell structure and very low switching energy, making it attractive for high-density, energy-constrained storage-class memory applications. In hierarchical memory system design, MRAM is often positioned closer to the CPU (cache/main-memory replacement candidate), while RRAM is well-suited for storage-class memory (SCM) positioned between DRAM and flash/SSD storage.